Extended outline for Phil’s Research approach

Note: This is not a repeat of what you wrote about in the document before you Qualifying exams, this is very much a “how to” document.

To design an experimental study for investigating wastewater treatment emissions of ammonia, hydrogen sulfide (H₂S), and struvite formation, both at bench scale and large scale, the following structured approach is recommended:

1. Objectives

Overall goal: Quantify emissions of ammonia and hydrogen sulfide during key wastewater treatment stages.

- Characterize conditions and frequency of struvite (MgNH₄PO₄·6H₂O) formation.

- Compare bench-scale results to large-scale (full plant) operations to assess emissions

  - Model emissions to better understand the chemistry

- Conduct a series of experiments to evaluate/optimize the chemical reactions

2. Experimental Design Overview

| Aspect | Bench Scale Study | Large Scale Study |
| --- | --- | --- |
| Scale | Bench/pilot reactors; controlled environment | Full process units at operational WWTP |
| Duration | Weeks to months (simulate seasonal variation) | multiple months (maybe seasonal) to capture operational variability |
| Replication | Multiple parallel reactors or lines | Multiple sampling locations and time points |
| Data Collection | High-frequency data (continuous monitors for automated, and manual sampling | Plant monitoring locations to assess temporal and spatial variation |

3. Key Variables and Parameters ( these are guess by me)

- Ammonia: Total ammonia, ammonium, pH, temperature, flow rate.

- Hydrogen Sulfide: Dissolved H₂S (liquid phase), gaseous H₂S (air phase), biofilm thickness, microbial activity.

- Struvite: pH, Mg²⁺, NH₄⁺, PO₄³⁻ concentrations, suspended solids, alkalinity, and mixing conditions.

4. Measurement Methods

Ammonia

- Airborne: Portable Ammonia gas data loggers

- Liquid phase: Use colorimetric methods, ion-selective electrodes, or spectrophotometry (Berthelot reaction).

- Calculate unionized ammonia: From total ammonia, pH, temperature, and ionic strength[3].

Hydrogen Sulfide

- Gas phase: Portable H₂S gas loggers, lead acetate tape monitors for ambient mapping[6].

- Liquid phase: Electrochemical or optical sensors for dissolved H₂S, preferred for process control as less affected by local turbulence].

Struvite

- Visual/physical: Inspect for scaling in reactors/pipes. (Historical data from your site and others)

- Chemical analysis: Measure Mg²⁺, NH₄⁺, PO₄³⁻, pH, and suspended solids; confirm struvite by XRD or SEM if possible (Data from literature or other WWTP, you do not need to do too much, just develop a metric for showing how the Struvite is reduced (% reduction) when Ammonia and H2S are controlled.

5. Experimental Procedures

A. Pilot Study

- Setup: Use lab-scale reactors or physical twins simulating real plant conditions (e.g., three parallel lines for robustness).  How many replicates do we need per condition to show Statistically significant results?  likely 6 data points at a minimum per experiments. We possibly can use a matrix of parameters and reduce the number of experiments.

- Control Variables: Independently vary hydraulic retention time, flow, Volume, C/N ratio, pH, and aeration, etc

- Sampling: Collect influent and effluent samples at regular intervals; monitor headspace and ambient air for H₂S and Ammonia.

- Struvite Monitoring: Track pH and key ion concentrations; periodically inspect for scale formation and collect precipitates for analysis.

B. Large Scale Study

- Site Selection: Choose representative units/locations (e.g., primary clarifiers, aeration basins, digesters). I am sure this is all in your head, but we need to lay it out on paper so we can discuss how many samples how long do we test, etc.

- Sampling Points: Install sensors for continuous ammonia and H₂S monitoring at critical process stages and emission points[12][5].

- Ambient Monitoring: Use spatial mapping  to assess H₂S & Ammonia dispersion around the plant].

- Struvite Management: Implement and monitor pH control strategies  to assess impact on struvite formation and scaling. Volume of HCl/unit of water (1000 gallons or some other appropriate volume)

6. Data Analysis

- Emission Rates: Calculate using concentration data, flow rates, and meteorological data for air emissions.

- Statistical Analysis: Use regression and response surface methodologies to identify key factors influencing emissions and struvite formation.

- Comparative Assessment: Benchmark pilot data against full-scale plant performance to validate scaling and process transferability.

7. Quality Assurance and Controls

- Calibration: Regularly calibrate all sensors and analytical instruments.

- Blanks and Standards: Use for all chemical analyses to ensure data reliability[3].

- Replicates: Ensure sufficient replication for statistical robustness. (how many samples or test?)

8. Reporting and Optimization

- Performance Metrics: Removal efficiencies, emission factors (e.g., g NH₃ or H2S/m³ sewage)[13], and struvite reduction.

- Operational Insights: Identify optimal conditions for minimizing emissions and controlling struvite (e.g., best pH, C/N ratio, aeration)

- Scale-up Recommendations: Use pilot findings to inform full-scale process design and operational protocols.

This design ensures robust, comparable data on ammonia, hydrogen sulfide, and struvite formation across both pilot and large-scale wastewater treatment operations, allowing for optimization and regulatory compliance.